Bumper Removal / Crash Bar Removal

1. Turn the wheel to full driver.

2. Remove 2x 15mm bolts, and loosen 2x bolts holding skid plate using impact driver (not wrench). Place Skid plate aside. (2 of the bolts don't need to be fully removed)

3. Remove 5x 11mm bolts from bottom of bumper using impact. (ADD site says 11mm, but if memory serves they weren't 11mm. I think 10mm?)

4. Remove 5x plastic snap plugs on top side of intercooler opening from bumper.

5. Pull off the front/lower plastic piece of the front bumper.

6. Remove 3x 21mm bumper mounting nuts on driver side with impact wrench.

7. Pull drivers side of bumper forward enough to clear crash bars.

8. Remove 2x 15mm bolts from top of driver's side crash bar using ratchet (there wasn't enough clearance for an impact.) Pull out crash bar.

9. Put bumper back into place and reinstall the 3x 21mm bumper mounting nuts with impact wrench. Use ratchet to verify torque.

10. Turn the wheel all the way passenger.

11. Remove 3x 21mm bumper mounting nuts on passenger side with impact wrench.

12. Pull passenger side of bumper forward enough to clear crash bars.

13. Remove 2x 15mm bolts from top of passenger side crash bar using ratchet. Remove Crash bar.

14. Put bumper back into place and reinstall the 3x 21mm bumper mounting nuts with impact wrench. Use ratchet to verify torque.

15. Reinstall front/lower plastic piece of the front bumper

16. Re-install 5x plastic snap plugs on top side of intercooler opening.

17. Re-install 5x 11mm bolts using impact driver. Verify torque with ratchet.

18. Lay on back with skid plate on top of me. Push skid plate into the 2 bolts that are still on the frame. Install the other 2 bolts that were removed, tighten using impact wrench. Verify torque with ratchet.