THIS TSB has been copied and pasted from the Ford Website but I’ve deleted some content related to other cars. You need to go to the Ford website and download the full TSB.

TECHNICAL SERVICE BULLETIN
10R80 Automatic Transmission - Harsh/Delayed Engagement And/Or Harsh/Delayed Shift - Non-Main Control Break-In Capable24-2046
22 February 2024

This bulletin supersedes 23-2350. Reason for update: revised Issue statement and Service Procedure

Model:

| 2019-2023 Ranger | Transmission/Transaxle: 10R80 |
| --- | --- |

Issue: Some 2019-2023 Ranger vehicles equipped with a 10R80 transmission may exhibit a harsh/delayed engagement and/or harsh/delayed shift, an illuminated MIL with DTC P0751, P0752, P0756, P0757, P0761, P0762, P0766, P0767, P0771, P0772, P2700, P2701, P2702, P2703, P2704, P2705, P2707, P2708, P0729, P0731, P0732, P0733, P0734, P0735, P0736, P076F, P07D9, P07F6 and/or P07F7 stored in the PCM or TCM. This may be due to the transmission solenoid ID strategy, sticking valves in the main control valve body and/or axial movement of the CDF clutch cylinder (7H351) sleeve. To correct the condition, follow the Service Procedure to identify and correct the condition.

Action: Follow the Service Procedure to correct the condition on vehicles that meet all of the following criteria:

• One of the following vehicles:

- 2019-2023 Ranger

• Equipped with a 10R80 transmission

• At least one of the following conditions:

- DTC P0751, P0752, P0756, P0757, P0761, P0762, P0766, P0767, P0771, P0772, P2700, P2701, P2702, P2703, P2704, P2705, P2707, P2708, P0729, P0731, P0732, P0733, P0734, P0735, P0736, P076F, P07D9, P07F6 and/or P07F7

- Harsh engagement

- Delayed engagement

- Harsh shift

- Delayed shift

Parts - All Vehicles - Main Control Overhaul

| Service Part Number | Quantity | Description | Unit of Issue | Piece Quantity |
| --- | --- | --- | --- | --- |
| W712658-S439 | 1 | Solenoid Retaining Plate Bolt | 4 | 2 |
| HL3Z-7G007-A | 6 | Solenoid Retaining Clips | 1 | 6 |
| JL1Z-7N134-A | 1 | Park Override Bolt (Expedition/Navigator) | 1 | 1 |
| XT-12-QULV | As Needed | Motorcraft® MERCON® ULV Automatic Transmission Fluid |  |  |

Parts - All Vehicles - Main Control Overhaul - Parts To Inspect And Replace Only If Necessary

| Service Part Number | Quantity | Description | Unit of Issue | Piece Quantity |
| --- | --- | --- | --- | --- |
| HL3Z-7A191-B | If Needed | Fluid Pan Gasket | 1 | 1 |
| L1MZ-7A098-A | If Needed | Fluid Filter (F-150/Expedition/Navigator/Mustang/Explorer) | 1 | 1 |
| HL3Z-7A098-A | If Needed | Fluid Filter (Ranger) | 1 | 1 |
| 7T4Z-7Z302-A | If Needed | Transmission Fluid Filter Seal | 1 | 1 |
| HL3Z-7J227-A | If Needed | Auxiliary Pump Tube O-ring (If Equipped) | 1 | 1 |
| HL3Z-7Z490-E | If Needed | Chanel Plate | 1 | 1 |
| HL3Z-7Z490-D | If Needed | Separator Plate (2017 F-150) | 1 | 1 |
| JL1Z-7Z490-C | If Needed | Separator Plate (Expedition/Navigator With Electronic Gearshift) | 1 | 1 |
| JL3Z-7Z490-E | If Needed | Separator Plate (Expedition/2018-2020 F-150 Without Electronic Gearshift, All Ranger) | 1 | 1 |
| JL3Z-7Z490-D | If Needed | Separator Plate (Mustang Built On Or Before 3-Jan-2021) | 1 | 1 |
| L1MZ-7Z490-J | If Needed | Separator Plate (Mustang Built On Or After 4-Jan-2021) | 1 | 1 |

Parts - All Vehicles - CDF Cylinder Replacement

| Service Part Number | Quantity | Description | Unit of Issue | Piece Quantity |
| --- | --- | --- | --- | --- |
| HL3Z-7G199-A | 1 | Auxiliary Pump Tube Seal (Stop/Start) | 1 | 1 |
| HL3Z-7A248-A | 1 | Torque Converter Seal | 1 | 1 |
| JL3Z-7N134-A | 12 | Front Support Bolts | 1 | 12 |
| LC3Z-7H223-A | 12 | Front Support Bolt Seals | 1 | 12 |
| HL3Z-7A248-G | 1 | Front Support To Case Seal | 1 | 1 |
| HL3Z-7G091-F | 5 | Input Shaft Seals (F2) | 1 | 5 |
| HL3Z-7B399-C | 4 | Sun Gear No. 3 Shaft Seals (F7) | 1 | 4 |
| HL3Z-7C099-A | 1 | C Clutch Balance Dam Inner Seal | 1 | 1 |
| HL3Z-7A548-B | 2 | C Clutch Balance Dam And Piston Outer Seal | 1 | 2 |
| HL3Z-7D404-A | 2 | C And D Clutch Piston Inner Seals | 1 | 2 |
| HL3Z-7A262-C | 1 | D Clutch Balance Dam | 1 | 1 |
| HL3Z-7D403-A | 1 | D Clutch Piston Outer Seal | 1 | 1 |
| HL3Z-7A548-G | 2 | F Clutch Balance Dam And Piston Outer Seal | 1 | 2 |
| HL3Z-7A548-A | 2 | F Clutch Balance Dam And Piston Inner Seal | 1 | 2 |
| HL3Z-7G091-G | 5 | Input Shaft To Sun Gear No. 3 Shaft Seals (F8) | 1 | 5 |
| HL3Z-7G091-C | 1 | Input Shaft Seal (F9) | 1 | 1 |
| JL3Z-7H351-B | 1 | CDF Cylinder (F-150/Expedition/Navigator, 5.0L Mustang) | 1 | 1 |
| JR3Z-7H351-B | 1 | CDF Cylinder (Ranger/2.3LMustang) | 1 | 1 |

Parts - All Vehicles - CDF Cylinder Replacement - Parts To Inspect And Replace Only If Necessary

| Service Part Number | Quantity | Description | Unit of Issue | Piece Quantity |
| --- | --- | --- | --- | --- |
| HL3Z-7B066-AB | If Needed | A Clutch Pressure Plate | 1 | 1 |
| HL3Z-7B164-E | If Needed | A Clutch Friction Plates | 1 | 3 |
| HL3Z-7F220-A | If Needed | A Clutch Steel Plates | 1 | 2 |
| HL3Z-7B442-F | If Needed | C Clutch Steel Plates | 1 | Ranger Require 4 Pieces, F-150/Mustang/Expedition/Navigator Require 5 Pieces |
| HL3Z-7B164-A | If Needed | C Clutch Friction Plates | 1 | Ranger Require 4 Pieces, F-150/Mustang/Expedition/Navigator Require 5 Pieces |
| ML3Z-7B477-A | If Needed | C Clutch Pressure Plate | 1 | 1 |
| HL3Z-7B442-D | If Needed | D Clutch Steel Plates | 1 | Ranger Require 5 Pieces, F-150/Mustang/Expedition/Navigator Require 6 Pieces |
| HL3Z-7B164-C | If Needed | D Clutch Friction Plates | 1 | Ranger Require 5 Pieces, F-150/Mustang/Expedition/Navigator Require 6 Pieces |
| HL3Z-7B066-E | If Needed | D Clutch Pressure Plate | 1 | 1 |
| HL3Z-7B164-G | If Needed | F Clutch Steel Plates | 1 | Ranger Require 3 Pieces, F-150/Mustang/Expedition/Navigator Require 4 Pieces |
| HL3Z-7B164-D | If Needed | F Clutch Friction Plates | 1 | Ranger Require 3 Pieces, F-150/Mustang/Expedition/Navigator Require 4 Pieces |
| HL3Z-7B066-A | If Needed | F Clutch Pressure Plate | 1 | 1 |

Parts - All Vehicles - CDF Cylinder Replacement - Select One Of The Following If Needed

| Service Part Number | Quantity | Description | Unit of Issue | Piece Quantity |
| --- | --- | --- | --- | --- |
| HL3Z-7B066-AA | If Needed | A Clutch Apply Plate 4.1 - 4.3 mm Selective | 1 | 1 |
| HL3Z-7B066-Z | If Needed | A Clutch Apply Plate 4.4 - 4.6 mm Selective | 1 | 1 |
| HL3Z-7B066-Y | If Needed | A Clutch Apply Plate 4.7 - 4.9 mm Selective | 1 | 1 |
| HL3Z-7B066-X | If Needed | A Clutch Apply Plate 5.0 - 5.2 mm Selective | 1 | 1 |
| HL3Z-7B066-W | If Needed | A Clutch Apply Plate 5.3 - 5.5 mm Selective | 1 | 1 |
| HL3Z-7H032-C | If Needed | T-3 Bearing (Replace If T-3 Shim Is Replaced) | 1 | 1 |
| HL3Z-7A527-Q | If Needed | T-3 Shim 3.05-3.15 mm Selective | 1 | 1 |
| HL3Z-7A527-P | If Needed | T-3 Shim 3.2-3.3 mm Selective | 1 | 1 |
| HL3Z-7A527-R | If Needed | T-3 Shim 3.35-3.45 mm Selective | 1 | 1 |
| HL3Z-7A527-K | If Needed | T-3 Shim 3.5-3.6 mm Selective | 1 | 1 |
| HL3Z-7A527-L | If Needed | T-3 Shim 3.65-3.75 mm Selective | 1 | 1 |
| HL3Z-7A527-M | If Needed | T-3 Shim 3.8-3.9 mm Selective | 1 | 1 |
| HL3Z-7A527-S | If Needed | T-3 Shim 3.95-4.05 mm Selective | 1 | 1 |
| HL3Z-7A527-T | If Needed | T-3 Shim 4.1-4.2 mm Selective | 1 | 1 |
| HL3Z-7A527-N | If Needed | T-3 Shim 4.25-4.35 mm Selective | 1 | 1 |
| HL3Z-7D483-A | If Needed | D Clutch Snap Ring 1.8 mm Selective | 1 | 1 |
| HL3Z-7D483-B | If Needed | D Clutch Snap Ring 2.0 mm Selective | 1 | 1 |
| HL3Z-7D483-C | If Needed | D Clutch Snap Ring 2.2 mm Selective | 1 | 1 |
| HL3Z-7D483-D | If Needed | D Clutch Snap Ring 2.4 mm Selective | 1 | 1 |
| HL3Z-7D483-E | If Needed | D Clutch Snap Ring 2.6 mm Selective | 1 | 1 |
| HL3Z-7D483-F | If Needed | D Clutch Snap Ring 2.8 mm Selective | 1 | 1 |
| HL3Z-7C122-A | If Needed | C Clutch Snap Ring 1.5 mm Selective | 1 | 1 |
| HL3Z-7C122-B | If Needed | C Clutch Snap Ring 1.7 mm Selective | 1 | 1 |
| HL3Z-7C122-C | If Needed | C Clutch Snap Ring 1.9 mm Selective | 1 | 1 |
| HL3Z-7C122-D | If Needed | C Clutch Snap Ring 2.1 mm Selective | 1 | 1 |
| HL3Z-7C122-E | If Needed | C Clutch Snap Ring 2.3 mm Selective | 1 | 1 |
| HL3Z-7C122-F | If Needed | C Clutch Snap Ring 2.5 mm Selective | 1 | 1 |
| HL3Z-7H365-C | If Needed | F Clutch Snap Ring 1.5 mm Selective | 1 | 1 |
| HL3Z-7H365-D | If Needed | F Clutch Snap Ring 1.7 mm Selective | 1 | 1 |
| HL3Z-7H365-E | If Needed | F Clutch Snap Ring 1.9 mm Selective | 1 | 1 |
| HL3Z-7H365-F | If Needed | F Clutch Snap Ring 2.1 mm Selective | 1 | 1 |
| HL3Z-7H365-G | If Needed | F Clutch Snap Ring 2.3 mm Selective | 1 | 1 |

Parts - Ranger Transmission Removal And Installation

| Service Part Number | Quantity | Description | Unit of Issue | Piece Quantity | Note |
| --- | --- | --- | --- | --- | --- |
| JB3Z-4B496-B | 6 | CV Joint-To-Transfer Case And Pinion Flange Bolts And Retaining Straps (4WD) | 1 | 6 |  |
| BE8Z-6731-AB | 1 | Engine Oil Filter And Gasket (4WD) | 1 | 1 |  |
| KB3Z-6L612-A | 1 | Catalytic Converter Gasket | 1 | 1 |  |
| W500121-S437 | 3 | Transfer Case Bolts (4WD) | 4 | 9 |  |
| W500642-S437 | 4 | Transmission Support Insulator Bolts (4WD) | 1 | 4 |  |
| W700056-S450B | 1 | Selector Lever Cable Bracket Bolts | 4 | 2 |  |
| W714265-S442 | 1 | Catalytic Converter Nuts | 4 | 3 |  |
| W715618-S437 | 2 | Torque Converter Nuts | 4 | 6 |  |
| W716936-S442 | 2 | Stabalizer Bar Bracket Nuts | 1 | 2 |  |
| W719427-S439 | 1 | Stabalizer Bar Bracket Bolts | 4 | 2 |  |
| W710233-S437 | 4 | Driveshaft Flange To Pinion Flange Bolts | 1 | 4 |  |
| W714780-S439 | 1 | Driveshaft Center Bearing Bolt (If Equipped With Two Piece Driveshaft) | 4 | 2 |  |
| W716344-S437 | 4 | Driveshaft Flange To Transmission Flange Bolts | 1 | 4 |  |
| PM-4-A | As Needed | Motorcraft® Metal Brake Parts Cleaner (Compliant With Low Volatile Organic Compound Requirements As Required In Some USA States) |  |  |  |
| PM-4-B | As Needed | Motorcraft® Metal Brake Parts Cleaner (Not Compliant With Volatile Organic Compound Requirements) |  |  |  |
| XL-2 | As Needed | Motorcraft® High Temperature Nickel Anti-Seize Lubricant |  |  |  |
| XL-5-A | As Needed | Motorcraft® Multi-Purpose Grease Spray |  |  |  |
| XO-5W30-Q1SP | As Needed | Motorcraft® SAE 5W-30 Synthetic Blend Motor Oil (All Markets Except Canada) |  |  |  |
| CXO-5W30-LSP6 | As Needed | Motorcraft® SAE 5W-30 Super Premium Motor Oil (Canada Only) |  |  |  |
| XT-10-QLVC | As Needed | Motorcraft® MERCON® LV Automatic Transmission Fluid (4WD) (All Markets Except Canada) |  |  | Transfer Case Fluid |
| CXT-10-LV6 | As Needed | Motorcraft® MERCON® LV Automatic Transmission Fluid (4WD) (Canada Only) |  |  | Transfer Case Fluid |
| XT-12-QULV | As Needed | Motorcraft® MERCON® ULV Automatic Transmission Fluid |  |  |  |

Parts - Ranger Transmission Removal And Installation - Parts To Inspect And Replace Only If Necessary

| Service Part Number | Quantity | Description | Unit of Issue | Piece Quantity |
| --- | --- | --- | --- | --- |
| JB3Z-7J227-A | If Needed | Transmission Fluid Cooler Tube Seals | 1 | 4 |
| W701183-S1300 | If Needed | Engine Block Dowel Pins (2.3L) | 4 | 2 |

Quantity refers to the amount of the service part number package(s) required to repair the vehicle.

Unit of Issue refers to the number of individual pieces included in a service part number package.

Piece Quantity refers to the total number of individual pieces required to repair the vehicle.

As Needed indicates the amount of the part may vary and/or is not a whole number. Parts can be billed out as non-whole numbers, including less than 1.

If Needed indicates the part is not mandatory.

Warranty Status: Information Only.

Repair/Claim Coding

| Causal Part: | IN |
| --- | --- |
| Condition Code: | 42 |

Service Procedure

NOTE: This article is for information only. Determine the causal part number and use available labor times in Section 7 of the SLTS Manual or claim M-time in accordance with the Warranty and Policy Manual. Causal part number IN in this article refers to the information only status and is not able to be claimed.

1. Using the latest software level of the appropriate Ford diagnostic scan tool, check for DTCs.

2. Are any of the following DTCs present? P0751, P0752, P0756, P0757, P0761, P0762, P0766, P0767, P0771, P0772, P2700, P2701, P2702, P2703, P2704, P2705, P2707, P2708, P0729, P0731, P0732, P0733, P0734, P0735, P0736, P076F, P07D9, P07F6 and/or P07F7.

(1). Yes - proceed to Sticking Valves - Main Control Valve Body Step 1.

(2). No - proceed to Transmission Solenoid Strategy Step 1.

Transmission Solenoid Strategy

1. Reprogram the existing transmission strategy download into the PCM/ TCM. Refer to WSM, Section 307-01.

(1). For FDRS applications when prompted, select Transmission hardware was not replaced - Reprogram TRID strategy for the existing transmission.

(2). For IDS applications when prompted, select Reprogram Only-No Hardware Replaced.

2. Perform the adaptive learning drive cycle. Refer to WSM, Section 307-01.

3. Does the vehicle still exhibit the condition after performing the adaptive learning drive?

(1). Yes - proceed to Sticking Valves - Main Control Valve Body, Step 1.

(2). No - repair is complete.

NOTE: Advise the customer this vehicle is equipped with an adaptive transmission shift strategy which allows the vehicle's computer to learn the transmission's unique parameters and improve shift quality. When the adaptive strategy is reset, the computer will begin a relearning process. This relearning process may result in firmer than normal upshifts and downshifts for several days.

Sticking Valves - Main Control Valve Body

NOTE: Affected vehicles are not main control break-in capable.

1. Overhaul (clean and inspect) the main control valve body and perform the adaptive learning drive cycle. Refer to WSM, Section 307-01.

2. Does the vehicle still exhibit the condition after overhauling the main control valve body and performing an adaptive learning drive cycle?

(1). Yes, vehicles built on or before 15-Aug-2022 - proceed to CDF Clutch Cylinder Replacement Step 1.

(2). Yes, vehicles built on or after 16-Aug-2022 - this article is complete. Refer to WSM, Section 307-01 for normal diagnostics.

(3). No - repair is complete.

NOTE: Advise the customer that this vehicle is equipped with an adaptive transmission shift strategy which allows the vehicle's computer to learn the transmission's unique parameters and improve shift quality. When the adaptive strategy is reset, the computer will begin a relearning process. This relearning process may result in firmer than normal upshifts and downshifts for several days.

CDF Clutch Cylinder Replacement

NOTE: For Mexico, USA and Canadian vehicles less than 3 years old and has less than 36,000 miles (60,000 km) the dealer has the option to continue with this TSB repair or replace the transmission under the low time in service (LTIS) policy.

1. Remove the transmission and mount the transmission to the bench. Refer to WSM, Section 307-01.

2. Disassemble the transmission. Perform only the necessary steps to remove the clutch and planetary assembly from the transmission case. Refer to WSM, Section 307-01.

(1). It is only necessary to remove the torque converter, transmission fluid pan and gasket, transmission fluid auxiliary pump (if equipped), fluid filter and main control valve body assembly, all 4 speed sensors (intermediate speed sensor A [ISSA], intermediate speed sensor B [ISSB], turbine shaft speed [TSS] and output shaft speed [OSS]), transmission fluid pump, front support assembly and the clutch and planetary assembly. Refer to WSM, Section 307-01.

3. Disassemble the clutch and planetary assembly. Perform only the necessary steps to remove the CDF clutch cylinder and the No. 3 sun gear shaft and No. 2 ring gear assembly from the clutch and planetary assembly. Refer to WSM, Section 307-01.

(1). It is only necessary to remove the A clutch assembly, the selective shim and T3 thrust bearing, remove and discard the 5-input shaft front seals.

(2). Remove the No. 1 planetary carrier snap ring, clutch, and planetary container cylinder, the E clutch and input shaft assembly, the No. 3 planetary carrier and No. 3 sun gear, the No. 3 sun gear shaft and No. 2 ring gear assembly. Refer to WSM, Section 307-01.

4. Remove and discard the sun gear No. 3 shaft seals. Install the 4 new sun gear No. 3 shaft seals. Refer to WSM, Section 307-01.

5. Disassemble the C, D and F clutch assemblies from the CDF cylinder. Discard the CDF cylinder. Refer to WSM, Section 307-01.

6. Assemble the C, D and F clutch assemblies into the new CDF clutch cylinder. Refer to WSM, Section 307-01.

7. Perform the C, D and F clutch pack endplay measurements for proper clearance. Refer to WSM, Section 307-01.

8. Remove and discard the input shaft-to-sun gear No. 3 shaft seals. Install the 5 new input shaft-to-sun gear No. 3 shaft seals. Refer to WSM, Section 307-01.

9. Remove and discard the input shaft seal. Install the new input shaft seal. Refer to WSM, Section 307-01.

10. Install the 5 new input shaft front seals. Refer to WSM, Section 307-01.

11. To reassemble the clutch and planetary assembly, reverse the disassembly procedure. Refer to WSM, Section 307-01.

12. Perform the T3 thrust bearing measurement to set transmission front end clearance. Refer to WSM, Section 307-01.

13. Reassemble the transmission. Refer to WSM, Section 307-01.

14. Install the transmission. Refer to WSM, Section 307-01.

15. Perform an adaptive learning drive cycle. Refer to WSM, Section 307-01.

NOTE: Advise the customer that this vehicle is equipped with an adaptive transmission shift strategy which allows the vehicle's computer to learn the transmission's unique parameters and improve shift quality. When the adaptive strategy is reset, the computer will begin a relearning process. This relearning process may result in firmer than normal upshifts and downshifts for several days.

© 2024 Ford Motor Company

All rights reserved.

NOTE: The information in Technical Service Bulletins is intended for use by trained, professional technicians with the knowledge, tools, and equipment to do the job properly and safely. It informs these technicians of conditions that may occur on some vehicles, or provides information that could assist in proper vehicle service. The procedures should not be performed by "do-it-yourselfers". Do not assume that a condition described affects your car or truck. Contact a Ford or Lincoln dealership to determine whether the Bulletin applies to your vehicle. Warranty Policy and Extended Service Plan documentation determine Warranty and/or Extended Service Plan coverage unless stated otherwise in the TSB article. The information in this Technical Service Bulletin (TSB) was current at the time of printing. Ford Motor Company reserves the right to supersede this information with updates. The most recent information is available through Ford Motor Company's on-line technical resources.